Robert Finch

A Proposed MMU core offering backwards compatibility with the 68851.

rf68851

Paged Memory Management Unit

# Goals

- Backwards compatibility.

  - Backwards compatibility is important to leverage existing software base.

- Support for 64-bit virtual and physical addresses.

  - Memory sizes in many cases exceed the address range of 32-bits.

- Support for larger page sizes.

  - As memory systems have grown it is more efficient to use a larger page size, 4kB and larger are popular with contemporary CPUs. For some purposes a MB page size is useful, but the 68851 only supports up to 32kB.

- Support for modern OS.

  - A modern OS may make use of more protection rings than the typical two ring scheme. Providing more protection rings eases the implementation of virtual machines.

- To be innovative rather than inventive.

  - An innovative approach takes an older design an renews it. Often older designs are very good, suited to the times they were created in. Being innovative provides an upgrade path for older designs.

- “Power-on” operation of the MMU.

  - The MMU should be able to translate addresses from a power-on state. Power on operation of the MMU may aid the security and integrity of the system.

## Root Pointers

The MMU contains five root pointer registers: the existing supervisor, CPU and DMA registers of the 68851 plus hypervisor and secure operating mode registers. Root pointer registers are extended to allow for up to 64-bit physical addresses.

| CRP / ARP | CPU (user/app) root pointer |
| --- | --- |
| SRP | Supervisor root pointer |
| HRP | Hypervisor root pointer |
| MRP | Secure machine root pointer |
| DRP | DMA root pointer |

### Root Pointer

Root pointer register supplies the entry point into the page tables as per the 68851.

|  | Root Pointer |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 63 | L/U | Limit14…0 |  |  |  |  |  |  |  |  |  |  | 48 |
| 47 | ~3 |  | ~3 | SG | ~ | ~ | ~ | ~ | ~ | ~ | ~ | DT2 | 32 |
| 31 | PA31…16 |  |  |  |  |  |  |  |  |  |  |  | 16 |
| 15 | PA15…4 |  |  |  |  |  |  |  |  | ~ | ~ | ~2 | 0 |

### Root Pointer Extension (RPX)

The root pointer extension supplies the upper 32-bits of a 64-bit physical address for the root pointer. It also supplies additional bits for the limit, allowing up to eight million entries in the root table. An additional bit is provided to extend the number of descriptor types to eight.

The default value of this register is zero except for the LVL field which is equal to two.

The default setting provides a high level of compatibility with the 68851 on reset.

|  | Root Pointer Extension |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 63 | PA63…48 |  |  |  |  |  | 48 |
| 47 | PA47…32 |  |  |  |  |  | 32 |
| 31 | LVL3 |  | RGN3 | ~4…0 |  | ~4…0 | 16 |
| 15 | DT | ~7 |  |  | Limit22…15 |  | 0 |

## Translation Control Registers

There is a separate pair (TC and XTC) of translation control registers for each operating mode. This allows page sizes to be different for each operating mode.

### Translation Control Register (TC)

|  | Translation Control Register |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31 | E | 05 |  | SRE | FCL | PS | IS | 16 |
| 15 | TIA |  | TIB |  |  | TIC | TID | 0 |

### Extended Translation Control Register (XTC)

The extended translation control register contains the number of logical address bits used for each table for table levels four through seven. The corresponding fields are ‘TIE’ to ’TIH’. The most significant bit of each field for ‘TIA’ through ‘TIH’ is stored in the fields labelled ‘A’ to ‘H’. In most cases the MSB will be zero.

|  | Extended Translation Control Register |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31 | A | B | C | D | E |  | SREX |  | F | G | M |  | PSX | H |  | ISX2 | 16 |
| 15 | TIE |  |  |  |  | TIF |  |  |  | TIG |  |  |  | TIH |  |  | 0 |

The ISX field adds two more bits to the IS field of the translation control register, allowing more of the upper address bits to be ignored during a translation.

Since the upper bits of a virtual address may indicate the read or write access level these bits will usually be ignored by the MMU.

PSX adds a bit onto the page size (PS) field allowing page sizes up to 2GB to be specified.

SREX adds a bit to the SRE bit forming a two-bit field. The two-bit field along with the operating mode indicates the root pointer selection.

The M field enables compatibility with the 68851. If M is set extra long format descriptors will not be recognized.

|  | {SREX,SRE} |  |  |  |
| --- | --- | --- | --- | --- |
| Operating Mode | 0 | 1 | 2 | 3 |
| 0 | CPU | CPU | CPU | CPU |
| 1 | CPU | SUPER | SUPER | SUPER |
| 2 | CPU | SUPER | HYPER | HPYER |
| 3 | CPU | SUPER | HYPER | SECURE |

On reset the extended translation control register is set to zero.

Page

| DT | Level 0 | Level 1 to 7 | Indirect Descriptor |
| --- | --- | --- | --- |
| Invalid 0 | Invalid | Invalid | Invalid |
| Page 1 | Type 1 page descriptor | Type 2 page descriptor | Type 1 page descriptor |
| Short 2 | Indirect | Table descriptor | Illegal |
| Long 3 | Indirect | Table descriptor | Illegal |
| Extra long 4 | Indirect | Table descriptor | Illegal |

# Descriptor Formats

## Existing table descriptor formats

For 64-bit physical addresses, the upper 32-bits come from the root pointer extension register. The short format table descriptor may not point to extra long descriptors.

|  | Short Format Table Descriptor |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 31 | PA31…16 |  |  |  | 16 |
| 15 | PA15…4 | U | WP | DT2 | 0 |

The rf68851 uses bits in the long format table descriptor that were previously defined to be zero.

The long format table descriptor has an extra bit for the descriptor type allowing the specification of extra long format descriptors.

A table level field was added.

|  | Long Format Table Descriptor |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 63 | L/U | Limit14…0 |  |  |  |  |  |  |  |  | 48 |
| 47 | RAL3 |  | WAL3 | SG | S | LVL3 | DT | U | WP | DT2 | 32 |
| 31 | PA31…16 |  |  |  |  |  |  |  |  |  | 16 |
| 15 | PA15…4 |  |  |  |  |  |  | ~ | ~ | ~2 | 0 |

## Additional table descriptor format for rf68851

|  | Extra Long Format Table Descriptor |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 127 | L/U | Limit14…0 |  |  |  |  |  |  |  |  |  |  |  | 112 |
| 111 | RAL7…5 |  | WAL7…5 |  | SG |  | S | LVL3 | DT | U |  | WP | DT2 | 96 |
| 95 | PA31…16 |  |  |  |  |  |  |  |  |  |  |  |  | 80 |
| 79 | PA15…4 |  |  |  |  |  |  |  |  | ~ |  | ~ | ~2 | 64 |
| 63 | PA63…48 |  |  |  |  |  |  |  |  |  |  |  |  | 48 |
| 47 | PA47…32 |  |  |  |  |  |  |  |  |  |  |  |  | 32 |
| 31 | ~3 |  |  | RGN3 |  | RAL4…0 |  |  |  |  | WAL4…0 |  |  | 16 |
| 15 | Limit30…15 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |

- The limit field has been extended to 31-bits allowing up to 231 entries per table.

- RAL, WAL fields have been extended to eight bits allowing finer grained access levels.

- An additional 32-bit has been added to the physical address, physical addresses up to 64-bits may be specified.

- The RGN field is a three-bit index into an eight-entry region table describing associated memory regions.

- The LVL field reflects the table level that the descriptor points to.

## Existing Page Descriptor Formats

For 64-bit physical addresses, the upper 32-bits come from the root pointer extension register.

|  | Short Page Descriptor (Type 1 & 2) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31 | PA31…16 |  |  |  |  |  |  |  | 16 |
| 15 | PA15…8 | G | CI | L | M | U | WP | DT2 | 0 |

|  | Long Page Descriptor – Type 1 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 63 | ~ |  |  |  |  |  |  |  |  |  |  | 48 |
| 47 | RAL3 | WAL3 | SG | S | G | CI | L | M | U | WP | DT2 | 32 |
| 31 | PA31…16 |  |  |  |  |  |  |  |  |  |  | 16 |
| 15 | PA15…4 |  |  |  |  |  |  |  | ~ | ~ | ~2 | 0 |

|  | Long Page Descriptor – Type 2 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 63 | L/U | Limit14…0 |  |  |  |  |  |  |  |  |  |  | 48 |
| 47 | RAL3 |  | WAL3 | SG | S | G | CI | L | M | U | WP | DT2 | 32 |
| 31 | PA31…16 |  |  |  |  |  |  |  |  |  |  |  | 16 |
| 15 | PA15…4 |  |  |  |  |  |  |  |  | ~ | ~ | ~2 | 0 |

## Additional Page Descriptor Format (rf68851)

|  | Extra Long Page Descriptor |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 127 | L/U |  | Limit14…0 |  |  |  |  |  |  |  |  |  |  |  |  |  | 112 |
| 111 | RAL7…5 |  |  | WAL7…5 |  | SG |  | S | G | CI | L | M | U |  | WP | DT2 | 96 |
| 95 | PA31…16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 |
| 79 | PA15…4 |  |  |  |  |  |  |  |  |  |  |  | ~ |  | ~ | ~2 | 64 |
| 63 | PA63…48 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 48 |
| 47 | PA37…31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 32 |
| 31 | ~ | CL2 |  |  | RGN3 |  | RAL4…0 |  |  |  |  |  |  | WAL4…0 |  |  | 16 |
| 15 | Limit30…15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |

## Existing Indirect Descriptors

|  | Short Indirect Descriptor |  |  |
| --- | --- | --- | --- |
| 31 | PA31…16 |  | 16 |
| 15 | PA15…2 | DT2 | 0 |

|  | Long Indirect Descriptor |  |  |
| --- | --- | --- | --- |
| 63 | ~16 |  | 48 |
| 47 | ~14 | DT2 | 32 |
| 31 | PA31…16 |  | 16 |
| 15 | PA15…2 | ~2 | 0 |

## Additional Extra Long Indirect Descriptor (rf68851)

|  | Extra Long Indirect Descriptor |  |  |  |
| --- | --- | --- | --- | --- |
| 127 | ~16 |  |  | 112 |
| 111 | ~14 | DT3 |  | 96 |
| 96 | ~16 |  |  | 80 |
| 79 | ~16 |  |  | 64 |
| 63 | PA63…48 |  |  | 48 |
| 47 | PA47…32 |  |  | 32 |
| 31 | PA31…16 |  |  | 16 |
| 15 | PA15…2 |  | ~2 | 0 |

## Existing Invalid Descriptors

|  | Short Invalid Descriptor |  |  |
| --- | --- | --- | --- |
| 31 | ~16 |  | 16 |
| 15 | ~14 | DT2 | 0 |

|  | Long Invalid Descriptor |  |  |
| --- | --- | --- | --- |
| 63 | ~16 |  | 48 |
| 47 | ~14 | DT2 | 32 |
| 31 | ~16 |  | 16 |
| 15 | ~16 |  | 0 |

## Additional Extra Long Invalid Descriptor (rf68851)

|  | Extra Long Invalid Descriptor |  |  |
| --- | --- | --- | --- |
| 127 | ~16 |  | 112 |
| 111 | ~14 | DT2 | 96 |
| 95 | ~16 |  | 80 |
| 79 | ~16 |  | 64 |
| 63 | ~16 |  | 48 |
| 47 | ~16 |  | 32 |
| 31 | ~16 |  | 16 |
| 15 | ~16 |  | 0 |

# Access Level Registers

The access level registers are extended by five bits, allowing finer grain access. The upper three bits of the register retain their original purpose. When the MMU is operating in a compatible mode the lower five bits of the register are ignored.

## CAL – Current Access Level

| 7 5 | 4 0 |
| --- | --- |
| CAL | CALX |

## VAL – Valid Access Level

| 7 5 | 4 0 |
| --- | --- |
| VAL | VALX |